Computer Architecture — Fall 2026

Project 1: Instruction Level Simulator — ISA Specification

Cheikh Lame, Ahmad Rao, Mira Allis, Randy Bauer

# Machine overview

- Word size: 32 bits. Every instruction is exactly 32 bits.

- Registers: 32 total, in two files of 16. Each 4-bit register field names one of 16 registers; the opcode (and sub-opcode) decides which file.

  - Integer registers R0–R15. R0 always reads as 0; writes to R0 are ignored. R15 is the link register: JAL saves the return address there.

  - Floating-point registers F0–F15. Each holds a 32-bit IEEE-754 single-precision value.

- Memory: instruction memory and data memory are separate.

  - Instruction memory holds one instruction per address. The PC counts instructions, not bytes. The first instruction in the assembly file is at address 0. Addresses are 28 bits.

  - Data memory is 2^28 bytes and byte addressable. A word occupies 4 bytes, stored big-endian (most significant byte at the lowest address). Word loads and stores must use an address that is a multiple of 4. An address outside 0 to 2^28 − 1 is an error; the simulator stops and reports it.

- PC update: every instruction performs PC ← PC + 1 unless its RTL assigns PC (taken branches, J, JAL, JR). HALT stops execution.

- Arithmetic: integers are two's complement. ADD, SUB, and ADDI wrap on overflow; nothing traps.

- Immediates: load/store offsets, branch offsets, and the ADDI immediate are sign-extended. The ANDI, ORI, and XORI immediates are zero-extended.

- Unused fields must be all zeros. So must the sub-opcode field of any instruction that does not use it.

# 32-bit instruction formats

R-format — register operations

| opcode | rs | rt | rd | sub-opcode | unused |
| --- | --- | --- | --- | --- | --- |
| 31–28 | 27–24 | 23–20 | 19–16 | 15–12 | 11–0 |
| 4 bits | 4 bits | 4 bits | 4 bits | 4 bits | 12 bits |

I-format — immediate, load/store, branch

| opcode | rs | rt | sub-opcode | immediate / offset |
| --- | --- | --- | --- | --- |
| 31–28 | 27–24 | 23–20 | 19–16 | 15–0 |
| 4 bits | 4 bits | 4 bits | 4 bits | 16 bits |

J-format — jumps

| opcode | address |
| --- | --- |
| 31–28 | 27–0 |
| 4 bits | 28 bits |

HALT format

| opcode | unused |
| --- | --- |
| 31–28 | 27–0 |
| 4 bits | 28 bits |

Every format totals exactly 32 bits.

# Opcode map

| Opcode | Instruction class | Format | Description |
| --- | --- | --- | --- |
| 0000 | ALU-R | R | Register-to-register integer ALU operations (sub-opcode selects the operation) |
| 0001 | ALU-I | I | Register-immediate integer ALU operations (sub-opcode selects the operation) |
| 0010 | LW | I | Load 32-bit integer |
| 0011 | SW | I | Store 32-bit integer |
| 0100 | BRANCH | I | Conditional branches (sub-opcode selects the comparison) |
| 0101 | J | J | Unconditional jump |
| 0110 | JAL | J | Jump and save return address |
| 0111 | JR | R | Jump to the address held in a register |
| 1000 | FLW | I | Load 32-bit floating-point value |
| 1001 | FSW | I | Store 32-bit floating-point value |
| 1010 | FP-ALU | R | Floating-point arithmetic, compare, and move (sub-opcode selects the operation) |
| 1011 | RESERVED | — | Available for future expansion |
| 1100 | LUI | I | Load upper immediate |
| 1101 | RESERVED | — | Available for future expansion |
| 1110 | RESERVED | — | Available for future expansion |
| 1111 | HALT | HALT | Stop execution |

# ALU-R operations (opcode 0000, R-format)

| Sub-opcode | Instruction | RTL | Description | Note |
| --- | --- | --- | --- | --- |
| 0000 | ADD rd, rs, rt | R[rd] ← R[rs] + R[rt] PC ← PC + 1 | Add | — |
| 0001 | SUB rd, rs, rt | R[rd] ← R[rs] - R[rt] PC ← PC + 1 | Subtract | — |
| 0010 | AND rd, rs, rt | R[rd] ← R[rs] AND R[rt] PC ← PC + 1 | Bitwise AND | Used to mask: keeps only the bits that are 1 in both. |
| 0011 | OR rd, rs, rt | R[rd] ← R[rs] OR R[rt] PC ← PC + 1 | Bitwise OR | Used to set selected bits to 1. |
| 0100 | XOR rd, rs, rt | R[rd] ← R[rs] XOR R[rt] PC ← PC + 1 | Bitwise exclusive OR | Flips selected bits. XOR rd, rs, rs sets rd to 0. |
| 0101 | SLT rd, rs, rt | R[rd] ← 1 if R[rs] < R[rt] (signed) else 0 PC ← PC + 1 | Set on less than, signed | Does not branch. Stores a 1 or 0 that a later BEQ/BNE can test against R0. |
| 0110 | SLTU rd, rs, rt | R[rd] ← 1 if unsigned(R[rs]) < unsigned(R[rt]) else 0 PC ← PC + 1 | Set on less than, unsigned | For values that are never negative, such as addresses and counts. |
| 0111 | SLL rd, rs, rt | R[rd] ← R[rs] << R[rt][4:0] PC ← PC + 1 | Shift left logical | Multiplies by a power of two. The shift amount comes from the low 5 bits of R[rt] (0–31). |
| 1000 | SRL rd, rs, rt | R[rd] ← R[rs] >> R[rt][4:0], zeros shifted in PC ← PC + 1 | Shift right logical | Divides an unsigned value by a power of two. |
| 1001 | SRA rd, rs, rt | R[rd] ← R[rs] >> R[rt][4:0], sign bit shifted in PC ← PC + 1 | Shift right arithmetic | Divides a signed value by a power of two; negative numbers stay negative. |

Sub-opcodes 1010–1111 are reserved.

# ALU-I operations (opcode 0001, I-format)

| Sub-opcode | Instruction | RTL | Description | Note |
| --- | --- | --- | --- | --- |
| 0000 | ADDI rt, rs, imm | R[rt] ← R[rs] + SignExt(imm) PC ← PC + 1 | Add immediate | Useful for loop counters and stepping pointers. With rs = R0 it loads a constant from -32,768 to 32,767. ADDI rt, rs, 0 copies a register. |
| 0001 | ANDI rt, rs, imm | R[rt] ← R[rs] AND ZeroExt(imm) PC ← PC + 1 | AND with immediate | Immediate is zero-extended, since a mask is never negative. |
| 0010 | ORI rt, rs, imm | R[rt] ← R[rs] OR ZeroExt(imm) PC ← PC + 1 | OR with immediate | Pairs with LUI to build a full 32-bit constant. |
| 0011 | XORI rt, rs, imm | R[rt] ← R[rs] XOR ZeroExt(imm) PC ← PC + 1 | XOR with immediate | Flips selected bits. |

Sub-opcodes 0100–1111 are reserved.

# BRANCH operations (opcode 0100, I-format)

All branches use the form Bxx rs, rt, offset. The offset counts instructions, measured from the instruction after the branch.

| Sub-opcode | Instruction | RTL | Description | Note |
| --- | --- | --- | --- | --- |
| 0000 | BEQ rs, rt, offset | if R[rs] = R[rt] then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if equal | BEQ rs, R0 branches if rs is zero. |
| 0001 | BNE rs, rt, offset | if R[rs] ≠ R[rt] then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if not equal | — |
| 0010 | BLT rs, rt, offset | if R[rs] < R[rt] (signed) then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if less than, signed | For "greater than," swap the operands: a > b is the same as b < a. |
| 0011 | BGE rs, rt, offset | if R[rs] ≥ R[rt] (signed) then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if greater than or equal, signed | For "less than or equal," swap the operands. |
| 0100 | BLTU rs, rt, offset | if unsigned(R[rs]) < unsigned(R[rt]) then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if less than, unsigned | For comparing addresses and counts. |
| 0101 | BGEU rs, rt, offset | if unsigned(R[rs]) ≥ unsigned(R[rt]) then PC ← PC + 1 + SignExt(offset) else PC ← PC + 1 | Branch if greater than or equal, unsigned | For comparing addresses and counts. |

Sub-opcodes 0110–1111 are reserved.

# Single-opcode instructions

| Opcode | Instruction | Format | RTL | Description | Note |
| --- | --- | --- | --- | --- | --- |
| 0010 | LW rt, offset(rs) | I | R[rt] ← M[R[rs] + SignExt(offset)] PC ← PC + 1 | Load a 32-bit integer from data memory | The address must be a multiple of 4. Sub-opcode field must be 0000. |
| 0011 | SW rt, offset(rs) | I | M[R[rs] + SignExt(offset)] ← R[rt] PC ← PC + 1 | Store a 32-bit integer to data memory | The address must be a multiple of 4. Sub-opcode field must be 0000. |
| 0101 | J address | J | PC ← address | Jump to an absolute instruction address | 28 address bits can reach every instruction. |
| 0110 | JAL address | J | R[15] ← PC + 1 PC ← address | Jump to a fixed address and save the return address in R15 | Starts a function call. The return address always goes to R15. A function that calls another function must save R15 to memory first, or it loses its own return address. |
| 0111 | JR rs | R | PC ← R[rs] | Jump to the address held in a register | JR R15 returns from a function. The rt, rd, and sub-opcode fields must be 0. |
| 1000 | FLW ft, offset(rs) | I | F[rt] ← M[R[rs] + SignExt(offset)] PC ← PC + 1 | Load a 32-bit floating-point value from data memory | The base address comes from an integer register; the destination is a floating-point register. The 32 bits are copied unchanged. |
| 1001 | FSW ft, offset(rs) | I | M[R[rs] + SignExt(offset)] ← F[rt] PC ← PC + 1 | Store a 32-bit floating-point value to data memory | The base address comes from an integer register; the source is a floating-point register. |
| 1100 | LUI rt, imm | I | R[rt] ← imm << 16 (lower 16 bits become 0) PC ← PC + 1 | Load upper immediate | Loads the upper half of a 32-bit constant. Follow with ORI rt, rt, lower16 to fill in the lower half. The rs and sub-opcode fields must be 0. |
| 1111 | HALT | HALT | Stop execution | End the program | Every benchmark must end with HALT. The simulator then prints instruction counts and writes the output files. |

# FP-ALU operations (opcode 1010, R-format)

The rs, rt, and rd fields name floating-point registers, except that FLT and FEQ write their result to an integer register R[rd].

| Sub-opcode | Instruction | RTL | Description | Note |
| --- | --- | --- | --- | --- |
| 0000 | FADD fd, fs, ft | F[rd] ← F[rs] + F[rt] PC ← PC + 1 | Floating-point addition | IEEE-754 single-precision arithmetic. |
| 0001 | FSUB fd, fs, ft | F[rd] ← F[rs] - F[rt] PC ← PC + 1 | Floating-point subtraction | — |
| 0010 | FMUL fd, fs, ft | F[rd] ← F[rs] × F[rt] PC ← PC + 1 | Floating-point multiplication | — |
| 0011 | FDIV fd, fs, ft | F[rd] ← F[rs] ÷ F[rt] PC ← PC + 1 | Floating-point division | Dividing by zero gives infinity or NaN as IEEE-754 defines; it does not stop the program. |
| 0100 | FLT rd, fs, ft | R[rd] ← 1 if F[rs] < F[rt] else 0 PC ← PC + 1 | Floating-point less-than compare | The result goes to an integer register so a branch can test it: FLT R1, F2, F3 then BNE R1, R0, target. Needed by the floating-point MAXFINDER. Any comparison with NaN gives 0. |
| 0101 | FEQ rd, fs, ft | R[rd] ← 1 if F[rs] = F[rt] else 0 PC ← PC + 1 | Floating-point equality compare | Result goes to an integer register, like FLT. |
| 0110 | FMOV fd, fs | F[rd] ← F[rs] PC ← PC + 1 | Copy a floating-point register | The rt field must be 0. There is no floating-point zero register, so this is the way to copy one FP register into another. |

Sub-opcodes 0111–1111 are reserved.

# Encoding examples

| Assembly | Fields | Hex |
| --- | --- | --- |
| ADD R3, R1, R2 | opcode 0000, rs 0001, rt 0010, rd 0011, sub-opcode 0000, unused 0 | 0x01230000 |
| BLT R2, R3, -4 | opcode 0100, rs 0010, rt 0011, sub-opcode 0010, offset 1111111111111100 | 0x4232FFFC |
| FLT R5, F3, F4 | opcode 1010, rs 0011, rt 0100, rd 0101, sub-opcode 0100, unused 0 | 0xA3454000 |

# RTL notation

| Symbol | Meaning |
| --- | --- |
| ← | Assignment. The right side is computed using values from before the instruction, then stored in the left side. |
| R[x] | Integer register x. R[0] always reads as 0. |
| F[x] | Floating-point register x. |
| M[a] | The 32-bit word stored in data memory bytes a through a+3, big-endian. |
| PC | Program counter: the instruction-memory address of the current instruction. |
| SignExt(v) | v widened to 32 bits by copying its top bit leftward, so negative values stay negative. |
| ZeroExt(v) | v widened to 32 bits by filling with zeros. |
| unsigned(x) | x read as a value from 0 to 2^32 − 1 instead of as a signed number. |
| x[4:0] | The low 5 bits of x. |
| << , >> | Shift left, shift right. |